ELEMENT VERIFICATION RESULTS

SSAD v1.0  ·  BS 8110-1:1997 / BS 5950-1:2000

| Document Title | Structural Element Calculation Verification Results |
| --- | --- |
| Application | SSAD — Smart Structural Analysis & Design (SSAD v1.0) |
| Design Standards | BS 8110-1:1997 / BS 5950-1:2000 |
| Date of Issue | 29 March 2026 |

Verification Methodology:  Each element module is tested with three independent sets of input parameters.  Reference values are derived from first-principle calculations using the applicable design standard and are compared against app-generated outputs with a tolerance of ±2 %.  A test case is marked VERIFIED if all numerical comparisons are within tolerance and the app returns a valid structural verdict.

# Executive Summary

Total test cases: 24     Verified: 24     Failed: 0

| # | Module | Standard | TC-1 | TC-2 | TC-3 |
| --- | --- | --- | --- | --- | --- |
| 1 | RC Beam | BS 8110-1:1997 | ✓ PASS | ✓ PASS | ✓ PASS |
| 2 | RC Slab | BS 8110-1:1997 | ✓ PASS | ✓ PASS | ✓ PASS |
| 3 | RC Column | BS 8110-1:1997 | ✓ PASS | ✓ PASS | ✓ PASS |
| 4 | RC Pad Footing | BS 8110-1:1997 | ✓ PASS | ✓ PASS | ✓ PASS |
| 5 | Steel Beam | BS 5950-1:2000 | ✓ PASS | ✓ PASS | ✓ PASS |
| 6 | Steel Column | BS 5950-1:2000 | ✓ PASS | ✓ PASS | ✓ PASS |
| 7 | Steel Truss Member | BS 5950-1:2000 | ✓ PASS | ✓ PASS | ✓ PASS |
| 8 | Steel Portal Frame | BS 5950-1:2000 | ✓ PASS | ✓ PASS | ✓ PASS |

# RC Beam  (BS 8110-1:1997)

Rectangular RC beam design — bending, shear and deflection checks.

## RC-B-01  ·  250×450 mm Simply Supported Beam — 5 m Span

Applicable Clause:  BS 8110-1:1997, Cl. 3.4.4 (Bending), Cl. 3.4.5 (Shear)

App Structural Verdict:  WARN

Verification Status:  VERIFIED ✓

Input Parameters:

| Parameter | Value |
| --- | --- |
| geometry.span | 5.0 |
| geometry.b | 250.0 |
| geometry.h | 450.0 |
| geometry.cover | 30.0 |
| geometry.support_type | simply_supported |
| materials.fcu | 30.0 |
| materials.fy | 460.0 |
| loads.gk | 10.0 |
| loads.qk | 7.5 |

Verification Results (Reference vs App, tolerance ±2%):

| Check / Parameter | Reference Value | App Value | Tolerance | Result |
| --- | --- | --- | --- | --- |
| Design UDL | 26.000 | 26.000 | ±2 % | ✓ MATCH |
| Design moment | 81.250 | 81.250 | ±2 % | ✓ MATCH |
| Design shear | 65.000 | 65.000 | ±2 % | ✓ MATCH |
| Effective depth | 404.000 | 404.000 | ±2 % | ✓ MATCH |
| K factor | 0.066 | 0.066 | ±2 % | ✓ MATCH |
| As required | 545.700 | 546.337 | ±2 % | ✓ MATCH |

## RC-B-02  ·  300×550 mm Continuous Beam — 7 m Span

Applicable Clause:  BS 8110-1:1997, Cl. 3.4.4, Cl. 3.5.2.3 (continuous bending)

App Structural Verdict:  PASS

Verification Status:  VERIFIED ✓

Input Parameters:

| Parameter | Value |
| --- | --- |
| geometry.span | 7.0 |
| geometry.b | 300.0 |
| geometry.h | 550.0 |
| geometry.cover | 30.0 |
| geometry.support_type | continuous |
| materials.fcu | 25.0 |
| materials.fy | 460.0 |
| loads.gk | 15.0 |
| loads.qk | 10.0 |

Verification Results (Reference vs App, tolerance ±2%):

| Check / Parameter | Reference Value | App Value | Tolerance | Result |
| --- | --- | --- | --- | --- |
| Design UDL | 37.000 | 37.000 | ±2 % | ✓ MATCH |
| Design moment | 155.900 | 155.918 | ±2 % | ✓ MATCH |
| Design shear | 77.700 | 77.700 | ±2 % | ✓ MATCH |
| Effective depth | 504.000 | 504.000 | ±2 % | ✓ MATCH |
| K factor | 0.082 | 0.082 | ±2 % | ✓ MATCH |
| As required | 859.000 | 860.025 | ±2 % | ✓ MATCH |

## RC-B-03  ·  200×400 mm Cantilever Beam — 3 m Span

Applicable Clause:  BS 8110-1:1997, Cl. 3.4.4, Cl. 3.4.6 (deflection)

App Structural Verdict:  FAIL

Verification Status:  VERIFIED ✓

Input Parameters:

| Parameter | Value |
| --- | --- |
| geometry.span | 3.0 |
| geometry.b | 200.0 |
| geometry.h | 400.0 |
| geometry.cover | 25.0 |
| geometry.support_type | cantilever |
| materials.fcu | 35.0 |
| materials.fy | 460.0 |
| loads.gk | 8.0 |
| loads.qk | 5.0 |

Verification Results (Reference vs App, tolerance ±2%):

| Check / Parameter | Reference Value | App Value | Tolerance | Result |
| --- | --- | --- | --- | --- |
| Design UDL | 19.200 | 19.200 | ±2 % | ✓ MATCH |
| Design moment | 86.400 | 86.400 | ±2 % | ✓ MATCH |
| Design shear | 57.600 | 57.600 | ±2 % | ✓ MATCH |
| Effective depth | 359.000 | 359.000 | ±2 % | ✓ MATCH |
| K factor | 0.096 | 0.096 | ±2 % | ✓ MATCH |
| As required | 684.100 | 684.206 | ±2 % | ✓ MATCH |

# RC Slab  (BS 8110-1:1997)

RC slab design — one-way, two-way and cantilever configurations.

## RC-S-01  ·  One-Way Simply Supported Slab — h=200 mm, 4 m Span

Applicable Clause:  BS 8110-1:1997, Cl. 3.5 (one-way slab bending & deflection)

App Structural Verdict:  PASS

Verification Status:  VERIFIED ✓

Input Parameters:

| Parameter | Value |
| --- | --- |
| geometry.slab_type | one_way |
| geometry.span | 4.0 |
| geometry.h | 200.0 |
| geometry.cover | 30.0 |
| geometry.support_type | simply_supported |
| materials.fcu | 30.0 |
| materials.fy | 460.0 |
| loads.gk | 3.0 |
| loads.qk | 4.0 |

Verification Results (Reference vs App, tolerance ±2%):

| Check / Parameter | Reference Value | App Value | Tolerance | Result |
| --- | --- | --- | --- | --- |
| Design UDL | 10.600 | 10.600 | ±2 % | ✓ MATCH |
| Design moment | 21.200 | 21.200 | ±2 % | ✓ MATCH |
| Effective depth | 164.000 | 164.000 | ±2 % | ✓ MATCH |
| K factor | 0.026 | 0.026 | ±2 % | ✓ MATCH |
| As required | 340.000 | 340.010 | ±2 % | ✓ MATCH |

## RC-S-02  ·  Two-Way Simply Supported Slab — h=150 mm, lx=3.5 m, ly=5.0 m

Applicable Clause:  BS 8110-1:1997, Cl. 3.5.3.3 & Table 3.14 (two-way slab)

App Structural Verdict:  FAIL

Verification Status:  VERIFIED ✓

Input Parameters:

| Parameter | Value |
| --- | --- |
| geometry.slab_type | two_way |
| geometry.lx | 3.5 |
| geometry.ly | 5.0 |
| geometry.h | 150.0 |
| geometry.cover | 25.0 |
| materials.fcu | 30.0 |
| materials.fy | 460.0 |
| loads.gk | 2.5 |
| loads.qk | 3.0 |

Verification Results (Reference vs App, tolerance ±2%):

| Check / Parameter | Reference Value | App Value | Tolerance | Result |
| --- | --- | --- | --- | --- |
| n (ULS load) | 8.300 | — | ±2 % | N/A |
| Effective depth | 119.000 | 119.000 | ±2 % | ✓ MATCH |
| Msx (short span) | 10.070 | 10.211 | ±2 % | ✓ MATCH |
| As short span | 222.400 | 225.696 | ±2 % | ✓ MATCH |

## RC-S-03  ·  Cantilever Slab — h=150 mm, 2 m Span

Applicable Clause:  BS 8110-1:1997, Cl. 3.5 (cantilever slab)

App Structural Verdict:  FAIL

Verification Status:  VERIFIED ✓

Input Parameters:

| Parameter | Value |
| --- | --- |
| geometry.slab_type | cantilever |
| geometry.span | 2.0 |
| geometry.h | 150.0 |
| geometry.cover | 25.0 |
| materials.fcu | 30.0 |
| materials.fy | 460.0 |
| loads.gk | 3.0 |
| loads.qk | 2.0 |

Verification Results (Reference vs App, tolerance ±2%):

| Check / Parameter | Reference Value | App Value | Tolerance | Result |
| --- | --- | --- | --- | --- |
| Design UDL | 7.400 | 7.400 | ±2 % | ✓ MATCH |
| Design moment | 14.800 | 14.800 | ±2 % | ✓ MATCH |
| Effective depth | 119.000 | 119.000 | ±2 % | ✓ MATCH |
| K factor | 0.035 | 0.035 | ±2 % | ✓ MATCH |
| As required | 326.900 | 327.125 | ±2 % | ✓ MATCH |

# RC Column  (BS 8110-1:1997)

Rectangular braced RC column — short column axial + bending capacity.

## RC-C-01  ·  300×300 mm Short Braced Column — N=500 kN, Mx=20 kN·m

Applicable Clause:  BS 8110-1:1997, Cl. 3.8.4 (short braced column)

App Structural Verdict:  PASS

Verification Status:  VERIFIED ✓

Input Parameters:

| Parameter | Value |
| --- | --- |
| geometry.b | 300.0 |
| geometry.h | 300.0 |
| geometry.cover | 30.0 |
| geometry.le_x | 3.0 |
| geometry.le_y | 3.0 |
| materials.fcu | 30.0 |
| materials.fy | 460.0 |
| loads.N_Ed | 500.0 |
| loads.Mx | 20.0 |
| loads.My | 0.0 |

Verification Results (Reference vs App, tolerance ±2%):

| Check / Parameter | Reference Value | App Value | Tolerance | Result |
| --- | --- | --- | --- | --- |
| Gross section area | 90000.000 | 90000.000 | ±2 % | ✓ MATCH |
| lex/h | 10.000 | 10.000 | ±2 % | ✓ MATCH |

## RC-C-02  ·  350×350 mm Short Braced Column — N=800 kN, Mx=50 kN·m

Applicable Clause:  BS 8110-1:1997, Cl. 3.8.4

App Structural Verdict:  PASS

Verification Status:  VERIFIED ✓

Input Parameters:

| Parameter | Value |
| --- | --- |
| geometry.b | 350.0 |
| geometry.h | 350.0 |
| geometry.cover | 30.0 |
| geometry.le_x | 4.5 |
| geometry.le_y | 4.5 |
| materials.fcu | 30.0 |
| materials.fy | 460.0 |
| loads.N_Ed | 800.0 |
| loads.Mx | 50.0 |
| loads.My | 0.0 |

Verification Results (Reference vs App, tolerance ±2%):

| Check / Parameter | Reference Value | App Value | Tolerance | Result |
| --- | --- | --- | --- | --- |
| Gross section area | 122500.000 | 122500.000 | ±2 % | ✓ MATCH |
| lex/h | 12.857 | 12.857 | ±2 % | ✓ MATCH |

## RC-C-03  ·  400×400 mm Short Braced Column — N=1200 kN, Mx=60 kN·m

Applicable Clause:  BS 8110-1:1997, Cl. 3.8.4

App Structural Verdict:  PASS

Verification Status:  VERIFIED ✓

Input Parameters:

| Parameter | Value |
| --- | --- |
| geometry.b | 400.0 |
| geometry.h | 400.0 |
| geometry.cover | 40.0 |
| geometry.le_x | 3.6 |
| geometry.le_y | 3.6 |
| materials.fcu | 35.0 |
| materials.fy | 460.0 |
| loads.N_Ed | 1200.0 |
| loads.Mx | 60.0 |
| loads.My | 0.0 |

Verification Results (Reference vs App, tolerance ±2%):

| Check / Parameter | Reference Value | App Value | Tolerance | Result |
| --- | --- | --- | --- | --- |
| Gross section area | 160000.000 | 160000.000 | ±2 % | ✓ MATCH |
| lex/h | 9.000 | 9.000 | ±2 % | ✓ MATCH |

# RC Pad Footing  (BS 8110-1:1997)

Isolated RC pad footing — bearing, bending, shear and punching checks.

## RC-F-01  ·  2.5×2.0 m Pad Footing — N_sls=400 kN, qa=150 kN/m²

Applicable Clause:  BS 8110-1:1997, Cl. 3.11 (pad footing)

App Structural Verdict:  PASS

Verification Status:  VERIFIED ✓

Input Parameters:

| Parameter | Value |
| --- | --- |
| geometry.L | 2.5 |
| geometry.B | 2.0 |
| geometry.h | 500.0 |
| geometry.cover | 75.0 |
| geometry.col_h | 300.0 |
| geometry.col_b | 300.0 |
| materials.fcu | 30.0 |
| materials.fy | 460.0 |
| loads.N_sls | 400.0 |
| loads.N_uls | 560.0 |
| loads.qa | 150.0 |
| loads.M_sls | 0.0 |

Verification Results (Reference vs App, tolerance ±2%):

| Check / Parameter | Reference Value | App Value | Tolerance | Result |
| --- | --- | --- | --- | --- |
| ULS upward pressure | 112.000 | 112.000 | ±2 % | ✓ MATCH |
| Effective depth | 417.000 | 417.000 | ±2 % | ✓ MATCH |

## RC-F-02  ·  2.0×2.0 m Pad Footing — N_sls=350 kN, qa=120 kN/m²

Applicable Clause:  BS 8110-1:1997, Cl. 3.11

App Structural Verdict:  PASS

Verification Status:  VERIFIED ✓

Input Parameters:

| Parameter | Value |
| --- | --- |
| geometry.L | 2.0 |
| geometry.B | 2.0 |
| geometry.h | 450.0 |
| geometry.cover | 75.0 |
| geometry.col_h | 300.0 |
| geometry.col_b | 300.0 |
| materials.fcu | 30.0 |
| materials.fy | 460.0 |
| loads.N_sls | 350.0 |
| loads.N_uls | 490.0 |
| loads.qa | 120.0 |
| loads.M_sls | 0.0 |

Verification Results (Reference vs App, tolerance ±2%):

| Check / Parameter | Reference Value | App Value | Tolerance | Result |
| --- | --- | --- | --- | --- |
| ULS upward pressure | 122.500 | 122.500 | ±2 % | ✓ MATCH |
| Effective depth | 367.000 | 367.000 | ±2 % | ✓ MATCH |

## RC-F-03  ·  3.0×2.5 m Pad Footing — N_sls=700 kN, qa=200 kN/m²

Applicable Clause:  BS 8110-1:1997, Cl. 3.11

App Structural Verdict:  PASS

Verification Status:  VERIFIED ✓

Input Parameters:

| Parameter | Value |
| --- | --- |
| geometry.L | 3.0 |
| geometry.B | 2.5 |
| geometry.h | 550.0 |
| geometry.cover | 75.0 |
| geometry.col_h | 400.0 |
| geometry.col_b | 400.0 |
| materials.fcu | 30.0 |
| materials.fy | 460.0 |
| loads.N_sls | 700.0 |
| loads.N_uls | 980.0 |
| loads.qa | 200.0 |
| loads.M_sls | 0.0 |

Verification Results (Reference vs App, tolerance ±2%):

| Check / Parameter | Reference Value | App Value | Tolerance | Result |
| --- | --- | --- | --- | --- |
| ULS upward pressure | 130.670 | 130.667 | ±2 % | ✓ MATCH |
| Effective depth | 467.000 | 467.000 | ±2 % | ✓ MATCH |

# Steel Beam  (BS 5950-1:2000)

Universal Beam design — section class, shear, moment and deflection.

## ST-B-01  ·  457×191×67 UB — 8 m Simply Supported, gk=10 qk=8 kN/m

Applicable Clause:  BS 5950-1:2000, Cl. 4.2 (bending & shear)

App Structural Verdict:  FAIL

Verification Status:  VERIFIED ✓

Input Parameters:

| Parameter | Value |
| --- | --- |
| section.A_cm2 | 85.5 |
| section.Iy_cm4 | 29400.0 |
| section.Wpl_cm3 | 1470.0 |
| section.Wel_cm3 | 1300.0 |
| section.hw_mm | 407.6 |
| section.tw_mm | 8.5 |
| section.tf_mm | 12.7 |
| section.bf_mm | 189.9 |
| geometry.span | 8.0 |
| geometry.support_type | simply_supported |
| geometry.lateral_restraint | full |
| materials.py | 275.0 |
| loads.gk | 10.0 |
| loads.qk | 8.0 |

Verification Results (Reference vs App, tolerance ±2%):

| Check / Parameter | Reference Value | App Value | Tolerance | Result |
| --- | --- | --- | --- | --- |
| Design UDL | 26.800 | 26.800 | ±2 % | ✓ MATCH |
| Design moment | 214.400 | 214.400 | ±2 % | ✓ MATCH |
| Shear capacity | 571.700 | 571.659 | ±2 % | ✓ MATCH |
| Moment capacity | 404.300 | 404.250 | ±2 % | ✓ MATCH |

## ST-B-02  ·  533×210×82 UB — 10 m Simply Supported, gk=12 qk=10 kN/m

Applicable Clause:  BS 5950-1:2000, Cl. 4.2

App Structural Verdict:  FAIL

Verification Status:  VERIFIED ✓

Input Parameters:

| Parameter | Value |
| --- | --- |
| section.A_cm2 | 104.0 |
| section.Iy_cm4 | 47500.0 |
| section.Wpl_cm3 | 2060.0 |
| section.Wel_cm3 | 1800.0 |
| section.hw_mm | 476.5 |
| section.tw_mm | 10.1 |
| section.tf_mm | 13.2 |
| section.bf_mm | 208.8 |
| geometry.span | 10.0 |
| geometry.support_type | simply_supported |
| geometry.lateral_restraint | full |
| materials.py | 275.0 |
| loads.gk | 12.0 |
| loads.qk | 10.0 |

Verification Results (Reference vs App, tolerance ±2%):

| Check / Parameter | Reference Value | App Value | Tolerance | Result |
| --- | --- | --- | --- | --- |
| Design UDL | 32.800 | 32.800 | ±2 % | ✓ MATCH |
| Design moment | 410.000 | 410.000 | ±2 % | ✓ MATCH |
| Shear capacity | 793.700 | 794.087 | ±2 % | ✓ MATCH |
| Moment capacity | 566.500 | 566.500 | ±2 % | ✓ MATCH |

## ST-B-03  ·  356×171×51 UB — 6 m Continuous, gk=8 qk=6 kN/m

Applicable Clause:  BS 5950-1:2000, Cl. 4.2 (continuous, mid-span M = 0.086wL²)

App Structural Verdict:  FAIL

Verification Status:  VERIFIED ✓

Input Parameters:

| Parameter | Value |
| --- | --- |
| section.A_cm2 | 64.9 |
| section.Iy_cm4 | 14100.0 |
| section.Wpl_cm3 | 896.0 |
| section.Wel_cm3 | 796.0 |
| section.hw_mm | 321.4 |
| section.tw_mm | 7.3 |
| section.tf_mm | 11.5 |
| section.bf_mm | 171.5 |
| geometry.span | 6.0 |
| geometry.support_type | continuous |
| geometry.lateral_restraint | full |
| materials.py | 275.0 |
| loads.gk | 8.0 |
| loads.qk | 6.0 |

Verification Results (Reference vs App, tolerance ±2%):

| Check / Parameter | Reference Value | App Value | Tolerance | Result |
| --- | --- | --- | --- | --- |
| Design UDL | 20.800 | 20.800 | ±2 % | ✓ MATCH |
| Design moment | 64.500 | 64.397 | ±2 % | ✓ MATCH |
| Moment capacity | 246.400 | 246.400 | ±2 % | ✓ MATCH |
| Shear capacity | 387.900 | 387.126 | ±2 % | ✓ MATCH |

# Steel Column  (BS 5950-1:2000)

H/I-section column under combined compression and bending.

## ST-C-01  ·  UC 203×203×60 — le=3.5 m, N=500 kN, Mx=30 kN·m

Applicable Clause:  BS 5950-1:2000, Cl. 4.7 (compression) & Cl. 4.8.3 (interaction)

App Structural Verdict:  PASS

Verification Status:  VERIFIED ✓

Input Parameters:

| Parameter | Value |
| --- | --- |
| section.A_cm2 | 76.4 |
| section.iy_cm | 8.96 |
| section.iz_cm | 5.21 |
| section.Wpl_y_cm3 | 652.0 |
| section.Wpl_z_cm3 | 305.0 |
| section.tf_mm | 14.2 |
| geometry.le_y | 3.5 |
| geometry.le_z | 3.5 |
| materials.py | 275.0 |
| loads.N_Ed | 500.0 |
| loads.Mx | 30.0 |
| loads.My | 0.0 |

Verification Results (Reference vs App, tolerance ±2%):

| Check / Parameter | Reference Value | App Value | Tolerance | Result |
| --- | --- | --- | --- | --- |
| Slenderness (major) | 39.060 | 39.062 | ±2 % | ✓ MATCH |
| Slenderness (minor) | 67.180 | 67.178 | ±2 % | ✓ MATCH |

## ST-C-02  ·  UC 254×254×73 — le=4.0 m, N=800 kN, Mx=50 kN·m

Applicable Clause:  BS 5950-1:2000, Cl. 4.7 & Cl. 4.8.3

App Structural Verdict:  PASS

Verification Status:  VERIFIED ✓

Input Parameters:

| Parameter | Value |
| --- | --- |
| section.A_cm2 | 93.1 |
| section.iy_cm | 11.07 |
| section.iz_cm | 6.46 |
| section.Wpl_y_cm3 | 992.0 |
| section.Wpl_z_cm3 | 465.0 |
| section.tf_mm | 14.2 |
| geometry.le_y | 4.0 |
| geometry.le_z | 4.0 |
| materials.py | 275.0 |
| loads.N_Ed | 800.0 |
| loads.Mx | 50.0 |
| loads.My | 0.0 |

Verification Results (Reference vs App, tolerance ±2%):

| Check / Parameter | Reference Value | App Value | Tolerance | Result |
| --- | --- | --- | --- | --- |
| Slenderness (major) | 36.130 | 36.134 | ±2 % | ✓ MATCH |
| Slenderness (minor) | 61.920 | 61.919 | ±2 % | ✓ MATCH |

## ST-C-03  ·  UC 152×152×37 — le=3.0 m, N=150 kN, Mx=10 kN·m

Applicable Clause:  BS 5950-1:2000, Cl. 4.7 & Cl. 4.8.3

App Structural Verdict:  PASS

Verification Status:  VERIFIED ✓

Input Parameters:

| Parameter | Value |
| --- | --- |
| section.A_cm2 | 47.1 |
| section.iy_cm | 6.54 |
| section.iz_cm | 3.87 |
| section.Wpl_y_cm3 | 309.0 |
| section.Wpl_z_cm3 | 149.0 |
| section.tf_mm | 11.5 |
| geometry.le_y | 3.0 |
| geometry.le_z | 3.0 |
| materials.py | 275.0 |
| loads.N_Ed | 150.0 |
| loads.Mx | 10.0 |
| loads.My | 0.0 |

Verification Results (Reference vs App, tolerance ±2%):

| Check / Parameter | Reference Value | App Value | Tolerance | Result |
| --- | --- | --- | --- | --- |
| Slenderness (major) | 45.870 | 45.872 | ±2 % | ✓ MATCH |
| Slenderness (minor) | 77.520 | 77.519 | ±2 % | ✓ MATCH |

# Steel Truss Member  (BS 5950-1:2000)

Individual truss member — tension and compression capacity checks.

## ST-T-01  ·  CHS 101.6×3.2 Tension Member — le=2.0 m, F_Ed=+100 kN

Applicable Clause:  BS 5950-1:2000, Cl. 4.6 (tension)

App Structural Verdict:  PASS

Verification Status:  VERIFIED ✓

Input Parameters:

| Parameter | Value |
| --- | --- |
| section.A_cm2 | 9.97 |
| section.i_min_cm | 3.47 |
| section.member_type | chs |
| geometry.le | 2.0 |
| materials.py | 355.0 |
| loads.F_Ed | 100.0 |

Verification Results (Reference vs App, tolerance ±2%):

| Check / Parameter | Reference Value | App Value | Tolerance | Result |
| --- | --- | --- | --- | --- |
| Tension capacity | 353.900 | 353.935 | ±2 % | ✓ MATCH |

## ST-T-02  ·  RHS 100×60×4 Compression Member — le=3.0 m, F_Ed=−60 kN

Applicable Clause:  BS 5950-1:2000, Cl. 4.7 (compression, strut curve a)

App Structural Verdict:  PASS

Verification Status:  VERIFIED ✓

Input Parameters:

| Parameter | Value |
| --- | --- |
| section.A_cm2 | 9.67 |
| section.i_min_cm | 2.13 |
| section.member_type | rhs |
| geometry.le | 3.0 |
| materials.py | 275.0 |
| loads.F_Ed | -60.0 |

Verification Results (Reference vs App, tolerance ±2%):

| Check / Parameter | Reference Value | App Value | Tolerance | Result |
| --- | --- | --- | --- | --- |
| Compression capacity | 86.800 | 86.804 | ±2 % | ✓ MATCH |

## ST-T-03  ·  Angle 100×65×8 Tension Member — le=1.5 m, F_Ed=+80 kN

Applicable Clause:  BS 5950-1:2000, Cl. 4.6 (tension)

App Structural Verdict:  PASS

Verification Status:  VERIFIED ✓

Input Parameters:

| Parameter | Value |
| --- | --- |
| section.A_cm2 | 12.8 |
| section.i_min_cm | 1.6 |
| section.member_type | angle |
| geometry.le | 1.5 |
| materials.py | 275.0 |
| loads.F_Ed | 80.0 |

Verification Results (Reference vs App, tolerance ±2%):

| Check / Parameter | Reference Value | App Value | Tolerance | Result |
| --- | --- | --- | --- | --- |
| Tension capacity | 352.000 | 352.000 | ±2 % | ✓ MATCH |

# Steel Portal Frame  (BS 5950-1:2000)

Single-bay symmetric portal frame — simplified elastic analysis.

## ST-P-01  ·  20 m Span, h=5 m Portal — Pinned Bases, gk=0.5 qk=0.6 kN/m²

Applicable Clause:  BS 5950-1:2000, Cl. 4.8.3 (elastic simplified analysis, pinned base)

App Structural Verdict:  FAIL

Verification Status:  VERIFIED ✓

Input Parameters:

| Parameter | Value |
| --- | --- |
| geometry.span | 20.0 |
| geometry.height | 5.0 |
| geometry.bay_width | 6.0 |
| geometry.pitch_deg | 6.0 |
| geometry.base_type | pinned |
| rafter_section.A_cm2 | 60.0 |
| rafter_section.I_cm4 | 20000.0 |
| rafter_section.Wpl_cm3 | 900.0 |
| rafter_section.iy_cm | 8.0 |
| rafter_section.iz_cm | 2.0 |
| column_section.A_cm2 | 76.0 |
| column_section.I_cm4 | 6000.0 |
| column_section.Wpl_cm3 | 650.0 |
| column_section.iy_cm | 8.0 |
| column_section.iz_cm | 3.0 |
| materials.py | 275.0 |
| loads.gk | 0.5 |
| loads.qk | 0.6 |

Verification Results (Reference vs App, tolerance ±2%):

| Check / Parameter | Reference Value | App Value | Tolerance | Result |
| --- | --- | --- | --- | --- |
| Design UDL (w_uls) | 9.960 | 9.960 | ±2 % | ✓ MATCH |
| Horizontal thrust | 99.600 | 99.600 | ±2 % | ✓ MATCH |
| Eaves moment | 498.000 | 498.000 | ±2 % | ✓ MATCH |

## ST-P-02  ·  15 m Span, h=4 m Portal — Fixed Bases, gk=0.4 qk=0.5 kN/m²

Applicable Clause:  BS 5950-1:2000, Cl. 4.8.3 (elastic simplified analysis, fixed base)

App Structural Verdict:  FAIL

Verification Status:  VERIFIED ✓

Input Parameters:

| Parameter | Value |
| --- | --- |
| geometry.span | 15.0 |
| geometry.height | 4.0 |
| geometry.bay_width | 5.0 |
| geometry.pitch_deg | 8.0 |
| geometry.base_type | fixed |
| rafter_section.A_cm2 | 50.0 |
| rafter_section.I_cm4 | 15000.0 |
| rafter_section.Wpl_cm3 | 750.0 |
| rafter_section.iy_cm | 7.5 |
| rafter_section.iz_cm | 2.0 |
| column_section.A_cm2 | 60.0 |
| column_section.I_cm4 | 4000.0 |
| column_section.Wpl_cm3 | 500.0 |
| column_section.iy_cm | 7.0 |
| column_section.iz_cm | 2.5 |
| materials.py | 275.0 |
| loads.gk | 0.4 |
| loads.qk | 0.5 |

Verification Results (Reference vs App, tolerance ±2%):

| Check / Parameter | Reference Value | App Value | Tolerance | Result |
| --- | --- | --- | --- | --- |
| Design UDL (w_uls) | 6.800 | 6.800 | ±2 % | ✓ MATCH |
| Horizontal thrust | 71.700 | 71.719 | ±2 % | ✓ MATCH |
| Eaves moment | 286.900 | 286.875 | ±2 % | ✓ MATCH |

## ST-P-03  ·  25 m Span, h=7 m Portal — Pinned Bases, gk=0.45 qk=0.55 kN/m²

Applicable Clause:  BS 5950-1:2000, Cl. 4.8.3 (elastic simplified analysis, pinned base)

App Structural Verdict:  FAIL

Verification Status:  VERIFIED ✓

Input Parameters:

| Parameter | Value |
| --- | --- |
| geometry.span | 25.0 |
| geometry.height | 7.0 |
| geometry.bay_width | 7.0 |
| geometry.pitch_deg | 5.0 |
| geometry.base_type | pinned |
| rafter_section.A_cm2 | 75.0 |
| rafter_section.I_cm4 | 30000.0 |
| rafter_section.Wpl_cm3 | 1200.0 |
| rafter_section.iy_cm | 9.0 |
| rafter_section.iz_cm | 2.5 |
| column_section.A_cm2 | 90.0 |
| column_section.I_cm4 | 8000.0 |
| column_section.Wpl_cm3 | 800.0 |
| column_section.iy_cm | 9.0 |
| column_section.iz_cm | 3.5 |
| materials.py | 275.0 |
| loads.gk | 0.45 |
| loads.qk | 0.55 |

Verification Results (Reference vs App, tolerance ±2%):

| Check / Parameter | Reference Value | App Value | Tolerance | Result |
| --- | --- | --- | --- | --- |
| Design UDL (w_uls) | 10.570 | 10.570 | ±2 % | ✓ MATCH |
| Horizontal thrust | 118.000 | 117.969 | ±2 % | ✓ MATCH |
| Eaves moment | 826.000 | 825.781 | ±2 % | ✓ MATCH |

# Verification Sign-Off

| Prepared by |  |
| --- | --- |
| Reviewed by |  |
| Date | 29 March 2026 |
| Document Status | DRAFT — Requires Chartered Engineer Review |